Type-A and CPU IC Card Reader

DLL Library Reference Manual

(Android 2.2  ISO14443 typeA )

2012-10-18

1. IC card reader API function set	2

1.1 System Function	2

1.1.1 open	2

1.1.2 close	2

1.1.3 getVersion	2

1.2 ISO14443 Type-A Function	3

1.2.1 MFRead	3

1.2.2 MFWrite	4

1.2.3 getCardSN	4

1.3.1 CPU_RATS	5

1.3.2 CPU_APDU	6

1.3.3 CPU_APDU	6

Appendix	7

# 1. IC card reader API function set

## 1.1 System Function

### 1.1.1 open

| Features | Open the specified serial port and baud rate |
| --- | --- |
| Function prototype | HANDLE open (String nComPath, int nBaudrate) |
| Description | Open the specified serial port and baud rate Input parameters: nCom: Serial port “/dev/ttyS0” nBaudrate: serial port baud rate (9600, 19200, 38400, 57600, 115200) Output parameters: none |
| Return value(s) | If successful the function returns the window handle Otherwise it returns 0 (INVALID_HANDLE_VALUE) |

### 1.1.2 close

| Features | Close the specified port |
| --- | --- |
| Function prototype | BOOL close (HANDLE commHandle) |
| Description | Close the specified window Input parameter: CommHandle Specifies the window handle Output parameters: none |
| Return value(s) | If successful the function returns 1, otherwise 0 (ERROR) |

### 1.1.3 getVersion

| Features | Read version information |
| --- | --- |
| Function prototype | int getVerion(byte[] ver); |
| Description | Send ISO14443A card writing instruction Input parameters: none Output parameters: If successful buffer, buffer: version information |
| Return value(s) | If successful the function returns 0, otherwise it returns an error code (pls. see appendix) |

## 1.2 ISO14443 Type-A Function

### 1.2.1 MFRead

| Features | Integrated card inventory, anti-collision, card selection, password verification, card readers and other operations, a card reader to complete the command operation. |
| --- | --- |
| Function prototype | int MFRead (HANDLE commHandle, int DeviceAddress,unsigned char mode, unsigned char blk_add, unsigned char num_blk,unsigned char *snr, unsigned char *buffer) |
| Description | ISO14443 A reader sent instructions Input parameters: commHandle - Serial handle DeviceAddress - Device Address To communicate with the device address number, ranging from 0-255 Read mode control mode 0x00: Idle Mode + Key A 0x01: All Mode + Key A 0x02: Idle Mode + Key B 0x03: All Mode + Key B blk_add To read start address block, ranging from 0--63 num_blk The number of blocks to be read length value, ranging from 1-4 (m1 card) snr 6 byte key Output parameters: snr If successful, the 4-byte card serial number (from low to high) buffer If successful, buffer [0-N]: Returns the data from the card (16 * num_blk bytes) If it fails, buffer [0]: Returns the state, the specific meaning of Annex. Note: You can only read the block of the sector, because each sector has its own independent key |
| Return value(s) | If successful the function returns 0, otherwise it returns an error code (pls. see appendix) |

### 1.2.2 MFWrite

| Features | Integrated card inventory, anti-collision, card selection, password verification, write card operation, a write command is completed card operation. |
| --- | --- |
| Function prototype | int MFWrite (HANDLE commHandle, int DeviceAddress,unsigned char mode, unsigned char blk_add, unsigned char num_blk, unsigned char *key, unsigned char *buffer) |
| Description | Send ISO14443 A written instruction card Input parameters: commHandle - Serial handle DeviceAddress - Device Address To communicate with the device address number, ranging from 0-255 write mode control mode 0x00: Idle Mode + Key A 0x01: All Mode + Key A 0x02: Idle Mode + Key B 0x03: All Mode + Key B blk_add Start to write the address block, ranging from 0--63 num_blk Write length value of the number of blocks, ranging from 1-4 (m1 card) key 6 byte key buffer Data to be written ((16 * num_blk bytes)) Output parameters: buffer If successful, buffer [0-3]: 4-byte card serial number (from low to high) If it fails, buffer [0]: Returns the state, the specific meaning of Annex. Note: You can only write block this sector, because each sector has its own independent key |
| Return value(s) | If successful the function returns 0, otherwise it returns an error code (pls. see appendix) |

### 1.2.3 getCardSN

| Features | Integrated card inventory, anti-collision, card selection, a command to read the card serial number to complete the operation |
| --- | --- |
| Function prototype | int getCardSN (HANDLE commHandle,int DeviceAddress, unsigned char mode,unsigned char cmd,unsigned char *flag, unsigned char *buffer) |
| Description | Input parameters: commHandle - Serial handle DeviceAddress - Device Address To communicate with the device address number, ranging from 0-255 mode 0x00: All mode 0x01: Idle Mode cmd 0x00: no need to perform halt instruction 0x01: reader instruction execution halt Output parameters: flag - single sign Kadoka 0x00 - detected a card 0x01 - detects multiple cards (called again to get another Uid). buffer If successful, buffer [0-3]: 4-byte card serial number (from low to high) If it fails, buffer [0]: Returns the state, the specific meaning of Annex. |
| Return value(s) | If successful the function returns 0, otherwise it returns an error code (pls. see appendix) |

### 1.3.1 CPU_RATS

| Features | RATS Operation |
| --- | --- |
| Function prototype | int CPU_RATS(int handle, int address, byte[] param, byte[] buff, byte[] retlen) |
| Description | Input parameters: handle - handle serial address - device address To communicate with the device address number, ranging from 0-255 param Type the command parameter Output parameters: buff If successful, RATS returned data If it fails, buffer [0]: Returns the state, the specific meaning of Annex. Retlen Returns the length of the data |
| Return value(s) | If successful the function returns 0, otherwise it returns an error code (pls. see appendix) |

### 1.3.2 CPU_APDU

| Features | APDU Operation |
| --- | --- |
| Function prototype | int CPU_APDU(int handle, int address, byte[] param, byte paramlen, byte[] buff, byte[] retlen) |
| Description | Input parameters: handle - window handle address - device address To communicate with the device address number, ranging from 0-255 Param Type the command parameter Paramlen enter the command parameter length Output parameters: buff If successful, the data returned APDU Retlen Returns the length of the data |
| Return value(s) | If successful the function returns 0, otherwise it returns an error code (pls. see appendix) |

### 1.3.3 CPU_APDU

| Features | Reset Antenna Operation |
| --- | --- |
| Function prototype | int CPU_RST_Ant(int handle, int address, byte[] buff, byte[] retlen) |
| Description | Input parameters: handle - window handle address - device address To communicate with the device address number, ranging from 0-255 Output parameters: buff If successful, the data returned APDU Retlen Returns the length of the data |
| Return value(s) | If successful the function returns 0, otherwise it returns an error code (pls. see appendix) |

# Appendix

API function return code (return value)

| Return | Description |
| --- | --- |
| 0x00 | It indicates that the command is executed successfully |
| 0x02 | Indicates the length of the received data does not match |
| 0x03 | It indicates the serial port failure |
| 0x04 | Serial not received any data showing |
| 0x05 | Indicates the device address does not match |
| 0x07 | It indicates the checksum is not correct |
| 0x0A | Represents the input parameters is incorrect, see description of the function |

Reader status return codes (S)

| Status | Description |
| --- | --- |
| 0x00 | Successful execution |
| 0x01 | Command execution failure (see specific instructions function) |
| 0x80 | Setting parameters indicating success |
| 0x81 | It represents the parameter setting failure |
| 0x82 | Communication timeout |
| 0x83 | Card does not exist |
| 0x84 | Card data receiving error |
| 0x85 | Input parameters or command format false |
| 0x87 | Indicating an unknown error |
| 0x89 | Parameter of the command or the command format error |
| 0x8A | An error occurred in the initialization block |
| 0x8B | Get the wrong serial number in the anti-collision process |
| 0x8C | No password authentication through |
| 0x8f | Input instruction code does not exist |
| 0x90 | Card does not support this command |
| 0x91 | Command format error |
| 0x92 | FLAG parameter represents the command is not supported OPTION mode |
| 0x93 | He said to operate BLOCK does not exist |
| 0x94 | Object to be operated has locked and can not be modified |
| 0x95 | Showing the locking operation is unsuccessful |
| 0x96 | A write operation is not successful |
| 0x89 | Flag in 15693 card is incorrect |

Example

1. Copy the text file libReaderAndroid.so and java ReaderAndroid.java to your own projects, they need to reside the same directory. Or it can not successfully compiled.

2. Reader Android class function calls in your own projects you can achieve the appropriate Features

try {

byte[] bPWS=new byte[6];

byte[] buf=new byte[30];

ReaderAndroid mSerialPort = new ReaderAndroid(new File("/dev/ttyS0"), 9600);

int handle = mSerialPort.getHandle();

mSerialPort.MFRead(handle,0,(byte)0x00,(byte)0x01,(byte)0x01,bPWS,buf);

mSerialPort.close(handle );

}

catch (SecurityException e){

// Exception Handling

}

catch (IOException e) {

// Exception Handling

}

catch (InvalidParameterException e) {

// Exception Handling

}